NON 2

entity non2 is

port ( e : in bit; s : out bit );

end non2;

architecture logic of non2 is

begin

s <= '1' when e = '0' else '0';

end logic;

*************

OU 3

entity ou3 is

port( e : in bit_vector (0 to 1);  s : out bit );

end ou3;

architecture abstract of ou3 is

begin

process (e)

begin

case e is

when "00" =>	s<='0';

when others =>	s<='1';

end case;

end process;

end abstract;

*************

MUX 2bv

entity mux2bv is

port(e0, e1, sel : in bit;	s : out bit);

end mux2bv ;

architecture behav of mux2bv is

begin

process (e0, e1, sel)

begin

if sel = '0' then s <= e0;

else s <= e1;

end if;

end process;

end behav;

*************

D latch init

entity dlatchinit1 is

port(D, clock, init : in bit;	Q : out bit);

end dlatchinit1 ;

architecture behav of dlatchinit1 is

begin

process (clock, D, init)

begin

if (init='1') then		Q <= '1';

end if;

if (clock='1') then		Q <= D;

end if;

end process;

end behav;

*************

D edge

entity dedge1 is

port(D, clock : in bit;	Q : out bit);

end dedge1 ;

architecture primitive of dedge1 is

begin

process

begin

wait until clock = '1';

Q <= D;

end process;

end primitive;